USER MANUAL

（BlackBerry Version）

## 1. Select the correct installation package

The program in the folder 【BIN】 is suitable for BlackBerry mobile phones.

## 2. Installation

Please use the official installation tools to install.

## 3. Operation

### 3.1 Set network

After installation, first do not start, set up the network and then access.

- In the "Options", the "application ", find "ipcamera" program, click "Edit Permissions."

- In the "MIDP groups" in the "Data Access", all set: Allowed.

- In the "MIDP groups" in the "Network Access", all set: session.

- In the "MIDP groups" in the "low-level network access, " all set: session.

- Save. Exit.

- In the "Options", the "Advanced Options" and find "TCP / IP" program, set APN.

### 3.2 Start the program

### 3.3 Set the parameters

- Address/Port: Fill in the address of information equipment; Here can be:

  - The internet ip address of the ipcamera, For example, 113.116.42.31, port:10540

  - The manufacturer assigned the domain name, the domain name using the jump method. For example, 002aaai.nwsvr.com, and port must be: 8888

  - Third-party domain name, which points to the camera's IP address in the internet. For example, b.airby.cn, port:10540

- Series:  Please select a B series or H series; if complete error, the camera can not be seen.

- Username:  the camera built-in login user name.

- Password:  the camera built-in login user password.

### 3.4 View the camera

B series are shown below:

You can control the head of the camera by clicking the LEFT, RIGHT, UP, DOWN directional buttons. You can select the resolution by checking “160*120”,”320*240” and “640*480” option buttons.

H series are shown below:

You can control the head of the camera by clicking the LEFT, RIGHT, UP, DOWN directional buttons.